# OVERVIEW

- Set up NCI and accessdev to run ACCESS

- Figure out what region you want to simulate

- Download Harvey’s suite to your accessdev

- Download required orography files

- Configure the suite to run your region

- Run the suite

## 1) Access setup

Need an account with accessdev through NCI (groups: hh5, access, ...?)
Need a met office UK account at https://code.metoffice.gov.uk/

Connecting to accessdev:

https://accessdev.nci.org.au/trac/wiki/GettingConnected 
need to ssh-keygen and ssh-copy-id between raijin and accessdev 
check-access-setup # run this to check connections

Store met office password in cached GPG agent thingy (WITHIN ACCESSDEV)

https://code.metoffice.gov.uk/trac/home/wiki/AuthenticationCaching/GpgAgent

mosrs-setup

mosrs-auth

mosrs-cache-password # caches access permission for 12 hours or so

Example downloading/running suites here: https://accessdev.nci.org.au/trac/wiki/GettingConnected

## 2) Determine extent of interest

Burn area and nearby topography should be covered in smallest high resolution nest

- First look at burn area and nearby topography of interest to get an idea of where you want to simulate

- Write down min and max latitude and longitude you want to cover, also the mid point of that area (I used QGIS)

- Work out how many grid boxes will be simulated (for model configuration). Note that highest resolution nests have been .0028 and/or .0009 degrees (you could do something different). NOTE MODEL LIMITATIONS ON PROCESSOR COUNTS AND GRID BOX COUNTS: total processes should be multiple of 48, and grid boxes should be 24 * the number of processes (longitudinally and latitudinally)

EG for KI:

MIDDLE : -35.84, 137.00
want left to right > 0.94 degrees (based on visual inspection of fire area and island)

left to right must be less than 2 degrees (to avoid missing SRTM area)
want bottom to top > .4 degrees (and must be less than 1.68 degrees)

338 < npts_x < 714 (at .0028 resolution)
143 < npts_y < 600 (at .0028)

npts needs to be nprocs * 24

14 < nprocs_x < 29

5 < nprocs_y < 25
Procs inner product needs to be divisible by 48:

nprocs_y, nprocs_x = 10,24 (240 = 48*5)

grid points need to be nprocs * 24:

npts_y, npts_x = 240, 576

## 3) Download Harvey's suite to your accessdev

- Connect to accessdev (may require extra tools like bash/cygwin and putty and xming if using windows to connect)

- Copy Harvey’s suite

- It is copied into ~/roses/au-XXXX

## FOR LINUX (x11 forwarding needs to be set up)
# (I have connections stored in ~/.ssh/config on both my laptop and in GADI)

ssh gadi

ssh access

mosrs-auth

## Downloading suite

# copy Harvey's suite:

rosie copy --prefix=au au-aa860

# vi editor will open here, can just type :wq to write and quite the file

# then answer yes to prompt

my .ssh/config file is as follows:

$ cat .ssh/config

Host         access

HostName     accessdev.nci.org.au

User         jwg574

ForwardX11   true

ForwardAgent true

Host         gadi

HostName     gadi.nci.org.au

User         jwg574

ForwardX11   true

ForwardAgent true

## 4) Download orography

30m data for kangaroo island

1) download 1 arc second (30m) resolution srtm topography info
2) upload and convert to netcdf

Example:

# Harvey says to keep structure equivalent to how he's done it: /g/data/en0/hxy548/srtm/30m/
1) get tiffs from https://earthexplorer.usgs.gov/ - need to register to usgs

0) log in to usgs system
a) open data tab (second tab)
b) search for srtm data
c) choose global 1-arc second srtm dataset
d) back to first tab, then use the map do draw a polygon around the area you want for your smallest nest
e) click results, then download individually as GEO tiff files

2) Upload to /g/data/en0/jwg574/srtm/30m/
3) convert to netcdf: (Script available at /g/data/en0/jwg574/srtm/)

Example translating tif to nc:

module load gdal

gdal_translate -of NetCDF s31_e150_1arc_v3.tif s31_e150_1arc_v3.nc

## 5) Suite config

start dumps: ask Harvey to create start dump for case study – fire should start ~6 hours after simulation starts

1) Decide start time and ignition time for simulation

When harvey makes it, it will be in

/g/data/en0/hxy548/ics/start-dump.yyyymmddHHMM
2) Create polygon(s), list of latlon points will go into fire.inp

- I used QGIS to draw roughly matching fire areas to my starting simulation time

- I then created nodes using Vector→Geometry Tools→ Extract Nodes

- Then I saved the nodes layer as a CSV with GEOMETRY as_YX

- They are stored in the order you draw them so it’s easiest to draw them counter clockwise for entry into fire.inp

3) fire.inp (/roses/au-aa929/app/um/file/fire.inp -> found using "find ./ -name fire.inp")

- Simulation start time

- geopath (where you put srtm)

- Constant Fuel flags 
(until we have fuel files sorted out)

- Polygons for fire
I created the strings in excel based on lat,lon columns, then pasted into the fire.inp document fixing up the indices and spacing

4) Rose edit config

cd roses/au-aa929

rose edit

  - Cycle points set to UTC start time matching Harvey's dumps

  - Name the location

  - Set resolution nests:
Use numbers determined in step 2

## 6) Getting run started

- Login to accessdev

- authenticate with mosrs

- go to suite directory

- run suite

  - It’s running on GADI, showing info on accessdev through a GUI (takes a few seconds to appear)

  - You can close the GUI and it will keep on running

  - Pause and Stop buttons will pause or kill the run on GADI

  - rose sgc will bring back the GUI if you’ve closed it

  - rose suite-clean clears run files from GADI (may need if Harvey updates config code)

  - rose suite-restart restarts a run if it was killed because GADI went down

## log into accessdev
ssh access
## connect to mosrs
mosrs-auth ## connect to mosrs to allow fcm_make to grab files from repository
## go to where the suite is located
cd roses/au-aa929
## run suite
rose suite-run ## start run

## checking output

Data pops out currently to GADI at /g/data/en0/jwg574/ACCESS-Fire/...

AWS may be available from http://www.bom.gov.au/climate/data/index.shtml